SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER ARCHITECTURE COURSE CODE: CSA12

COURSE OUTCOME COVERED

CO1: Analyze the functional units of a computer system including CPU, memory, bus  structures, and I/O components by examining instruction execution cycles, addressing  modes, and performance metrics such as CPI, clock cycles, and benchmarking (BL4) Assessment Tool Weightage: 40%

QUESTIONS:5 Total Marks:25

ANNEXURE A

APPLICATION BASED QUESTIONS

Code of Conduct:

I Sakthi Priya J 192411110 (Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student: Sakthi Priya J

ANNEXURE A

- A smart city surveillance system processes continuous video feeds from multiple cameras, where  large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours,  the system experiences lag and delayed response in threat detection. Analyze how the interaction  between CPU, memory, and I/O contributes to this issue, and explain how different bus structures  (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system  performance.

- A real-time stock trading application running on a processor executes millions of instructions per  second, but users report delays in transaction processing during high load. The system has a high  CPI due to frequent memory access and branch instructions. Examine how the fetch–decode– execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and  improve overall CPU performance using suitable architectural enhancements.

- An embedded system based on an 8085 microprocessor is used in an industrial temperature control  unit, where sensors provide input and actuators respond accordingly. However, incorrect  temperature readings are occasionally processed due to improper use of addressing modes in the  program. Discuss how different addressing modes influence data access, identify possible causes  of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure  accurate and reliable operation.

- A computing system uses an 8086 microprocessor with segmented memory for executing multiple  programs simultaneously, but it encounters issues like incorrect data access and stack overflow  errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how  improper segment handling can lead to such problems, and explain how instruction execution and  addressing modes must be managed to ensure correct program execution.

- A data communication system represents packet information in hexadecimal format for ease of  debugging and transmission, but a software engineer mistakenly interprets values, leading to  incorrect data processing. Analyze the importance of number system conversions between  hexadecimal and binary in such systems, explain how errors in conversion can affect instruction  execution, and discuss how efficient CPU design and benchmarking can help detect and minimize  such issues in real-time systems.

MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understan ding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| Problem Solving Approach | 20 | Logical, well structured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, well organized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |

Answers

3.8085 Microprocessor – Addressing Modes in an Industrial Temperature Control Unit

Introduction

An 8085 microprocessor is an 8-bit processor widely used in embedded systems for industrial automation, monitoring, and control. In a temperature control unit, the microprocessor receives temperature values from sensors, processes the data, compares it with a preset value, and controls actuators such as heaters or cooling fans.

If the program uses an incorrect addressing mode, the processor may read the wrong memory location or incorrect sensor value, resulting in inaccurate temperature control.

Application

Industrial Temperature Control System

Applications include:

- Chemical industries

- Food processing plants

- Boiler temperature monitoring

- Cold storage systems

- Smart HVAC systems

Working:

- Temperature sensor measures temperature.

- Sensor data is stored in memory/I/O port.

- 8085 reads the data.

- It compares the temperature with the preset limit.

- Heater or cooling fan is switched ON/OFF.

Addressing Modes and Their Influence on Data Access

| Addressing Mode | Description | Example | Influence |
| --- | --- | --- | --- |
| Immediate | Data is given directly in instruction | MVI A,32H | Fast access, no memory lookup |
| Register | Data stored in register | MOV A,B | Very fast, used for calculations |
| Direct | Address is specified in instruction | LDA 2050H | Reads exact sensor memory location |
| Register Indirect | Register pair stores address | MOV A,M | Flexible access to sensor arrays |
| Implied | Operand is implied | CMA | No operand needed |

Possible Causes of Incorrect Temperature Readings

1. Wrong Direct Address

Example:

LDA 2051H

instead of

LDA 2050H

The processor reads another memory location instead of the sensor value.

2. Incorrect Register Pair

If HL points to the wrong address,

MOV A,M

reads invalid data.

3. Immediate Addressing Used Incorrectly

Using

MVI A,30H

loads constant 30H instead of reading the actual sensor value.

4. Register Overwritten

If another instruction changes register contents before comparison,

MOV B,A

may store incorrect data.

5. Memory Corruption

Sensor values may be overwritten accidentally due to programming mistakes.

Using the 8085 Instruction Set for Reliable Operation

Use the following instructions carefully:

| Instruction | Purpose |
| --- | --- |
| LDA | Read sensor data from memory |
| STA | Store processed data |
| MOV | Transfer data between registers |
| MVI | Load constants |
| CMP | Compare temperature with limit |
| JC | Jump if below limit |
| JZ | Jump if equal |
| JMP | Continue program |
| IN | Read sensor through input port |
| OUT | Control heater or fan |

Good programming practices:

- Use correct memory addresses.

- Initialize registers before use.

- Verify sensor data before processing.

- Use proper branching instructions.

- Keep sensor values protected in memory.

Example

Assume

Sensor temperature stored at 2050H

Preset temperature = 30°C (1EH)

Algorithm

- Read temperature.

- Compare with 30°C.

- If temperature is greater than 30°C

  - Turn ON cooling fan.

- Else

  - Turn ON heater.

Flowchart

START

Read Sensor Value

from 2050H

Compare with Preset (30°C)

Temp > 30°C?               No

Yes

Turn ON Fan     Turn ON Heater

STOP

8085 Assembly Logic

assembly

LDA 2050H

CPI 1EH

JC HEATER

MVI A,01H

OUT 01H

JMP STOP

HEATER:

MVI A,00H

OUT 01H

STOP:

HLT

Equivalent C Program

#include <stdio.h>

int main() {

int temperature = 35;

int preset = 30;

if (temperature > preset)

printf("Cooling Fan ON\n");

else

printf("Heater ON\n");

return 0;

}

Output

If

temperature = 35

Output:

Cooling Fan ON

If

temperature = 25

Output:

Heater ON

Conclusion

The addressing modes of the 8085 microprocessor determine how data is accessed from registers, memory, or immediate values. In an industrial temperature control system, selecting the correct addressing mode ensures that the processor reads the correct sensor data and controls the actuators accurately. Incorrect addressing can lead to wrong temperature readings and improper operation of heaters or cooling fans. By using appropriate addressing modes along with instructions such as LDA, MOV, CPI, JC, IN, and OUT, the system becomes accurate, reliable, and efficient for industrial temperature monitoring and control.

4.8086 Microprocessor – Segmentation and Addressing Modes in a Multitasking Computing System

Introduction

The 8086 microprocessor is a 16-bit processor that uses segmented memory architecture to access up to 1 MB of memory. Instead of using a single memory space, it divides memory into Code Segment (CS), Data Segment (DS), Stack Segment (SS), and Extra Segment (ES). This segmentation allows multiple programs and data to coexist efficiently.

In a multitasking system, improper handling of segments can cause incorrect data access, stack overflow, and program execution errors. Therefore, correct use of segmentation, addressing modes, and instruction execution is essential for reliable operation.

Application

Multitasking Computing System

Applications include:

- Multi-user operating systems

- Banking software

- Industrial automation systems

- Embedded control systems

- Real-time monitoring systems

Working:

- Program instructions are stored in the Code Segment (CS).

- Variables and data are stored in the Data Segment (DS).

- Function calls and return addresses are stored in the Stack Segment (SS).

- The CPU fetches instructions, processes data, and manages the stack during execution.

8086 Memory Segmentation

| Segment | Register | Purpose |
| --- | --- | --- |
| Code Segment | CS | Stores program instructions |
| Data Segment | DS | Stores variables and data |
| Stack Segment | SS | Stores stack, return addresses, local variables |
| Extra Segment | ES | Used for additional data storage |

Physical Address Calculation

Physical Address = (Segment × 10H) + Offset

Example:

- CS = 2000H

- IP = 1500H

Physical Address:

2000H × 10H = 20000H

20000H + 1500H = 21500H

How Improper Segment Handling Causes Problems

1. Incorrect Data Access

If the DS register points to the wrong memory segment, the processor reads incorrect data.

Example:

assembly

MOV AX,[1000H]

If DS is incorrect, the wrong memory location is accessed.

2. Stack Overflow

Continuous PUSH operations without corresponding POP operations fill the stack memory, causing stack overflow.

Example:

assembly

PUSH AX

PUSH BX

PUSH CX

Without POP instructions, the stack grows beyond its allocated space.

3. Wrong Code Execution

If the CS register contains an incorrect segment address, the CPU fetches invalid instructions, resulting in unexpected program behavior or crashes.

4. Improper Stack Segment

If SS is not initialized correctly, function calls (CALL) and returns (RET) fail because return addresses are stored in the wrong location.

Addressing Modes in 8086

| Addressing Mode | Description | Example |
| --- | --- | --- |
| Immediate | Data is part of instruction | MOV AX,1234H |
| Register | Data stored in register | MOV AX,BX |
| Direct | Memory address given directly | MOV AX,[2000H] |
| Register Indirect | Register holds memory address | MOV AX,[BX] |
| Based | Base register used | MOV AX,[BP+04H] |
| Indexed | Index register used | MOV AX,[SI] |
| Based Indexed | Base + Index registers | MOV AX,[BX+SI] |

Managing Instruction Execution Correctly

To ensure reliable program execution:

- Initialize CS, DS, and SS properly.

- Use correct addressing modes for accessing memory.

- Balance every PUSH instruction with a corresponding POP.

- Verify segment registers before accessing memory.

- Use CALL and RET correctly to maintain stack integrity.

Commonly used instructions:

| Instruction | Purpose |
| --- | --- |
| MOV | Transfer data |
| PUSH | Store data on stack |
| POP | Retrieve data from stack |
| CALL | Call a subroutine |
| RET | Return from subroutine |
| JMP | Jump to another instruction |
| LEA | Load effective address |

Example

Suppose:

- Code Segment stores program instructions.

- Data Segment stores two numbers.

- Stack Segment stores return addresses during function calls.

The processor reads two numbers, adds them, and stores the result.

Flowchart

8086 Assembly Program

assembly

MOV AX,DATA

MOV DS,AX

MOV AX,[1000H]

ADD AX,[1002H]

MOV [1004H],AX

HLT

Equivalent C Program

#include <stdio.h>

int main() {

int num1 = 20;

int num2 = 15;

int result;

result = num1 + num2;

printf("Sum = %d\n", result);

return 0;

}

Output

Sum = 35

Conclusion

The 8086 microprocessor uses memory segmentation to organize program instructions, data, and stack into separate segments, allowing efficient execution of multiple programs. Incorrect handling of Code Segment (CS), Data Segment (DS), or Stack Segment (SS) can lead to wrong data access, invalid instruction execution, and stack overflow errors. By properly initializing segment registers, using suitable addressing modes, and correctly managing instructions such as MOV, PUSH, POP, CALL, and RET, reliable and error-free program execution can be achieved.

1.Computer Organization – CPU, Memory, I/O Interaction and Bus Structure in a Smart City Surveillance System

Introduction

A smart city surveillance system continuously captures video from multiple cameras and processes it for object detection, traffic monitoring, and threat identification. The CPU, memory, and I/O devices work together to transfer and process large amounts of data. During peak hours, simultaneous video streams increase the workload, causing lag, memory access delays, and slow threat detection. Efficient bus structures and an optimized instruction execution cycle help improve overall system performance.

Application

Smart City Surveillance System

Applications include:

- Traffic monitoring

- Crime detection

- Face recognition

- Parking management

- Public safety monitoring

- Industrial security surveillance

Working:

- Cameras capture continuous video.

- I/O devices transfer video data to memory.

- CPU fetches video frames from memory.

- CPU processes the frames to detect threats.

- Results are displayed or alarms are generated.

Interaction Between CPU, Memory, and I/O

| Component | Function |
| --- | --- |
| CPU | Processes video data and detects threats |
| Memory (RAM) | Temporarily stores video frames and program data |
| I/O Devices | Cameras, storage devices, network interfaces |

Data Flow

Camera → I/O Device → Memory → CPU → Memory → Display / Alarm

Problem During Peak Hours

- Multiple cameras send data simultaneously.

- CPU waits for data from memory.

- Memory becomes busy handling many requests.

- I/O devices compete to transfer data.

- This results in delayed threat detection and reduced system performance.

Bus Structures

1. Single-Bus Structure

- CPU, memory, and I/O devices share the same bus.

- Only one data transfer occurs at a time.

- Creates bus congestion when many devices access the bus.

Advantages

- Simple design

- Low cost

Disadvantages

- Slow performance

- Bus bottleneck

- High waiting time

2. Multi-Bus Structure

- Separate buses are provided for CPU, memory, and I/O.

- Multiple transfers occur simultaneously.

- Reduces congestion and improves speed.

Advantages

- Faster data transfer

- Reduced CPU waiting time

- Better performance for large data processing

Disadvantages

- Higher cost

- More complex hardware

Comparison of Bus Structures

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of Buses | One | Multiple |
| Data Transfer | One at a time | Simultaneous |
| Speed | Low | High |
| Bus Congestion | High | Low |
| Cost | Low | High |
| Performance | Moderate | Excellent |

Improving the Instruction Execution Cycle

The CPU executes instructions in four stages:

- Fetch – Retrieve instruction from memory.

- Decode – Interpret the instruction.

- Execute – Perform the required operation.

- Store – Save the result back to memory.

Performance Improvements

- Use instruction pipelining to overlap instruction stages.

- Increase cache memory to reduce memory access time.

- Use Direct Memory Access (DMA) for high-speed I/O transfers.

- Implement interrupts instead of continuous polling.

- Use multi-bus architecture to reduce CPU waiting time.

These improvements reduce lag and increase threat detection speed.

Example

Suppose:

- 20 surveillance cameras continuously send video.

- Each camera transfers frames to RAM.

- CPU processes each frame for object detection.

Using a single bus, all cameras compete for the same communication path, causing delays.

Using a multi-bus system, cameras, memory, and CPU communicate simultaneously, improving response time and reducing lag.

Flowchart

START

Capture Video from Cameras

Transfer Data through I/O Devices

Store Video Frames in Memory

CPU Fetches and Processes Frames

Threat Detected?

Yes                  No

Generate Alarm        Continue Monitoring

STOP

C Program (Simulation)

#include <stdio.h>

int main() {

int cameras = 20;

int frames = 500;

printf("Number of Cameras : %d\n", cameras);

printf("Frames Processed  : %d\n", frames);

printf("Threat Detection Completed Successfully\n");

return 0;

}

Output

Number of Cameras : 20

Frames Processed  : 500

Threat Detection Completed Successfully

Conclusion

A smart city surveillance system depends on efficient interaction between the CPU, memory, and I/O devices to process continuous video streams. During peak hours, a single-bus architecture can become a bottleneck due to bus contention, resulting in lag and delayed threat detection. A multi-bus architecture, along with improvements such as instruction pipelining, cache memory, DMA, and interrupt-based I/O, significantly enhances system performance by reducing waiting time, increasing data transfer speed, and ensuring faster and more reliable threat detection.